Geologic and Karst Features Map of the Hagerstown Quadrangle,

Washington County, Maryland

By David K. Brezinsk,

Maryland Geological Survey

July, 2013

Lithologic description of geologic units, Hagerstown quadrangle.

Quateranry –

Alluvium (Qa)

Poorly sorted, unconsolidated, tan, reddish brown, to dark gray mud, silt, sand, and pebbles.  Deposited within the channels of streams and on the flood plain adjacent to the streams.  Thickness estimated at 3 to 10 feet (l to 3 m).

Terrace deposits (Qt)

Reddish brown to brown, sandy and clayey mixture of rounded pebbles to cobbles of sandstone, vein quartz, and quartzite.  Present along elevated areas above Antietam Creek.  Thickness ranges from a thin veneer to more than 10 feet (0 to 3 m).

#### Ordovician –

#### Rockdale Run Formation (Orr)

Interbedded and cyclic limestone and dolomite, cherty in the lower 400 feet (120 m).  Limestone intervals consist of medium to light gray, ribbony and thrombolitic to stromatolitic, lime mudstone to boundstone.  Locally, limestone layers are light gray oolitic packstone to oolitic grainstone.  Dolomite parts of cycles vary from tan, laminated, to light gray to tan, massive, fractured with wispy dolomitic laminae.  The relative proportion of the limestone to dolomite varies upsection.  In the lower 600 feet (180 m), limestone is typically thicker than dolomite. This progressively changes upsection so that the upper 700 feet (215 m) is dominantly dolomitic with little limestone within individual cycles.  Forms very little topographic expression; many horizons are very poorly exposed.  Thickness is up to 2,500 feet (760 m).

#### Stonehenge Formation

The Stonehenge Formation was subdivided into three separate members.  Only the lowest, the Stoufferstown Member, is named the other two are informal.

##### Upper member (Osu)

Medium to medium dark gray, medium-bedded, ribbony and oolitic, lime mudstone to packstone.  Near the base of the member ribbony lime mudstone predominates.  Upsection, medium gray, ribbony lime mudstone becomes interbedded with intervals of flat-pebble lime grainstone, and hummocky, thickly laminated lime packstone and oolitic lime packstone to grainstone.  Locally, thin (< 3.0 feet, 0.9 m), algal thrombolites are present.  This member commonly forms a persistent and mappable ridge and is frequently well-exposed. Thickness is 500 to 750 feet (150 to 215 m).

##### Middle member (Osm)

The lower part of the middle member is composed of massive, medium gray, algal lime boundstone with some layers up to 25 feet (7 m) thick.  Grades upsection into interbedded medium to dark gray algal thrombolites 5 to 7 feet (1.5 to 2.0 m) thick and medium gray, thinly bedded to ribbony, locally fossiliferous, lime wackestone to lime packstone.  Several thin, tan dolomite beds occur near the middle of the unit.  Thickness: 300 to 400 feet (90 to 100 m).

### Stoufferstown Member (Oss)

Dark gray, argillaceous, thinly bedded to ribbony, lime mudstone with thin beds of flat-pebble lime grainstone conglomerate and hummocky, discontinuous, thin beds of laminated limestone.  A single, 10-foot (3 m) interval of massive, dark gray, thrombolitic, algal boundstone occurs approximately 30 feet (10 m) above the base of the member.  This member weathers into thin, brown and orange chips, which litter overlying soil.  Forms a low, discontinuous ridge.  Thickness varies from 230 to 295 feet (70 to 90 m).

Cambrian

Conococheague Formation

Interbedded gray limestone and tan dolomite cycles.  Subdivided and mapped as three members, two of which are informal.  Total thickness is from 2,000 to 2,500 feet (610 to 760 m).

##### Upper member (Ccu)*

Interbedded medium to light gray, ribbony, lime mudstone that weathers to flaggy to platy beds, and arenaceous grainstone exhibiting edgewise and flat-pebble conglomerates.  Locally, thin, pastel blue and pink marble strata are developed.  Black or gray chert fragments and brown-weathering quartz sandstone cobbles are frequently abundant in overlying soil.  Thickness: 650 to 750 feet (200 to 230 m).

Middle member (Ccm)*

Predominantly cyclically bedded, medium to dark gray, thrombolitic limestone and gray, ribbony and laminated limestone and tan, laminated dolomite.  Thrombolites range in thickness from 3 to 6 feet (1 to 2 m) within thrombolitic intervals to less than 1 foot (0.3 m) within the ribbony intervals.  Several dark gray, oolitic intervals are present in the upper part of this member.  Thickness ranges from 1,500 to 1,800 feet (460 to 550 m).

Big Spring Station Member (Ccb)*

Tan, massive dolomite interbedded with tan to light gray, laminated dolomite; unit is characterized by dark brown weathering.  At the type section on the western side of the Hagerstown Valley, this member is characterized by interbeds of light gray, cross-bedded, calcareous, intraclastic, quartzarenitic sandstone approximately 3 feet (1 m) in thickness.  Member becomes interbedded with gray, dolomitic thrombolite beds near the top.  Thickness ranges from 200 to 300 feet (60 to 90 m).

Elbrook Formation (Ce)*

Lower part of the formation is very poorly exposed and contains interbedded tan, thin- to thick-bedded limestone and dolomite, which frequently weather shaly, and are intercalated with medium-bedded, dark gray limestone.  Middle part of the formation contains cyclic, dark gray limestone and dolomitic limestone.  The upper part of the formation, and making up its greatest thickness, is cyclically bedded, gray, thrombolitic limestone and ribbony to laminated limestone and dolomite.  Thickness ranges from 2,200 to 2,500 feet (670 to 760 m).

* The geologic symbol for Cambrian appears on the geologic map as